STM32 PERIPHERAL DRIVER ANALYSIS

GPIO :  GPIO stands for general purpose input output. This is used to interface external devices such as LEDs, sensors and displays with microcontrollers. In this project, the GPIO driver provides APIs that make pin configuration and control easier without directly modifying registers.

GPIO initialization

GPIO_init() – This function is used to initialize a GPIO pin.

*) Before confiquring a pin, the GPIO peripheral clock is enabled using the RCC reg.

Parameters are.

- Pin number

- Pin mode (input, output, alternate func, analog)

- Output type(push-pull or open drain)

- Speed

- Pull-up/pull-down resistor

Registers are: MODER, OTYPER, OSPEEDER and PUPDR .

INPUT AND OUTPUT OPERATIONS

The driver provides APIs to read and write GPIO pins.

- GPIO_ReadFromInputPin() – is used to read the state of a pin

- GPIO_WriteToOutputPin()- is used to set a pin HIGH or LOW.

- GPIO_ToggleOutputPin() – is used to toggle the current state of a pin.

FOR EXAMPLE:

The onboard LED connected to PA5 was controlled using gpio driver. The user push button connected to PC13 was read and LED was toggled when the button is pressed.

CONCLUSION:

The GPIO driver provides a simple interface for configuring and controlling GPIO pins. It reduces the programming complexity and to improve readability.