USART DRIVER ANALYSIS

INTRODUCTION:

USART stands for universal synchronous asynchronous receiver and transmitter. Is used for serial communication between the microcontroller and external devices. The USART driver provides APIs for data transmission , reception and interrupt handling without requiring direct access to USART registers.

APIs

Clock Control API

USART_PericlockControl():

Function: void USART_PericlockControl(USART_RegDef_t *pUSARTx, uint8_t EnorDi);

Description:

This function enables or disables the clock supply for the selected USART peripheral.

Before using USART, the peripheral clock must be enabled through the RCC (Reset and Clock Control) register.

- pUSARTx: Pointer to USART peripheral (USART1, USART2, etc.)

- EnorDi: Enables or disables the USART clock.

Initialization APIs

- USART_Init()

Function: void USART_Init(USART_Handle_t *pUSARTHandle);

Description:

This function initializes the USART peripheral according to the configuration provided in the USART handle structure.

Registers :

- CR1 Register: control register 1

- CR2 Register: control register 2

- BRR : baud rate register

- USART_DeInit()

Function: void USART_DeInit(USART_Handle_t *pUSARTHandle);

Description:

This function resets the selected USART peripheral and returns it to the default configuration state.

Data Transmission and Reception APIs

- USART_SendData()

Function: void USART_SendData( USART_Handle_t *pUSARTHandle, uint8_t *pTxBuffer, uint32_t Len);

Description: This function transmits data through USART.

- USART_ReceiveData()

Function: void USART_ReceiveData( USART_Handle_t *pUSARTHandle, uint8_t *pRxBuffer, uint32_t  Len);

Description: This function receives data from the USART peripheral.

Interrupt Based Communication APIs

- USART_SendDataIT()

Function: uint8_t USART_SendDataIT( USART_Handle_t *pUSARTHandle, uint8_t *pTxBuffer, uint32_t  Len);

Description: This function performs USART transmission using interrupts.

- USART_ReceiveDataIT()

Function: uint8_t USART_ReceiveDataIT( USART_Handle_t *pUSARTHandle, uint8_t *pRxBuffer,

uint32_t  Len );

Description: This function receives data using interrupts.

Interrupt APIs

- USART_IRQInterruptConfig()

Function: void USART_IRQInterruptConfig(uint8_t IRQNumber, uint8_t EnorDi);

Description: Enables or disables USART interrupts through the NVIC controller.

- USART_IRQPriorityConfig()

Function: void USART_IRQPriorityConfig(uint8_t IRQNumber, uint32_t IRQPriority);

Description: Sets the priority level for USART interrupts.

- USART_IRQHandling()

Function: void USART_IRQHandling(USART_Handle_t *pUSARTHandle);

Description: Handles USART interrupt events .

OTHER APIs

- USART_GetFlagStatus()

Function: uint8_t USART_GetFlagStatus( USART_RegDef_t *pUSARTx, uint8_t StatusFlagName);

Description: Checks the status of USART flags.

- USART_ClearFlag()

Function: void USART_ClearFlag( USART_RegDef_t *pUSARTx, uint16_t StatusFlagName );

Description: Clears USART status flags after handling communication events.

- USART_PeripheralControl()

Function: void USART_PeripheralControl( USART_RegDef_t *pUSARTx, uint8_t EnorDi );

Description: Enables or disables the USART peripheral.

- USART_SetBaudRate()

Function: void USART_SetBaudRate( USART_RegDef_t *pUSARTx, uint32_t BaudRate );

Description: Configures the USART communication speed by updating the baud rate register.